GUIDE

## SPECIFICATION

MANUFACTURER:

4Front Engineered Solutions – Kelley

US Corporate Office

# 1612 Hutton Drive, Suite 140

# Carrollton, Texas  75006

(800) 558-6960 voice

- (972) 323-2661 facsimile

www.kelleycompany.com

SECTION 11 13 19.33

Vehicle restraints – manual sURFACE-chock

**********************************************************************************************************************************

This guide specification section has been prepared by 4Front Engineered Solutions – Kelley and is intended to assist the specifier in preparing a specification for a manually operated truck restraint devices, complete with all available options and accessories.

Edit entire section to suit project requirements.  Modify or add items as necessary.  Delete items which are not applicable.  Words and sentences within brackets [_____] reflect a choice to be made regarding inclusion or exclusion of a particular item or statement.  This section may include performance, proprietary, and descriptive type specifications.  Edit to avoid conflicting requirements.

This guide specification is written around the Construction Specifications Institute (CSI), Section Format, and standard references to section names and numbers are based on the CSI Master Format 2004.

For specification assistance on specific product applications or information about other 4Front Engineered Solutions - Kelley products, please contact our offices above or any of our local product representatives throughout the country.

4Front Engineered Solutions - Kelley reserves the right to modify these guide specifications at any time.  Updates to this guide specification will be posted to the manufacturer’s web site and/or in printed matter as they occur.  4Front Engineered Solutions - Kelley makes no expressed or implied warranties regarding content, errors, or omissions in the information presented.

**********************************************************************************************************************************

- PART 1 - GENERAL

  - SUMMARY

    - Related Documents: Provisions established within General and Supplementary Conditions of the Contract, Division 1 - General Requirements and Drawings are collectively applicable to this Section.

    - Related Sections:

      - Section 05 55 00 - Metal Fabrications:  [Perimeter guard rails and inserts.] [Dock slab and door protection edge angles.]

      - Division 16 - Electrical:  Connections to dock equipment.

  - SYSTEM DESCRIPTION

    - Design Requirements: Provide fixed-in-place wheel engagement truck restraint devices.

    - Provide equipment which has been manufactured, fabricated, and installed to withstand loads specified and to maintain performance criteria stated by manufacturer without defects or failure.

    - Comply with (Material Handling Institute) ANSI MH30.3.

  - SUBMITTALS

    - General:  Submit in accordance with Section 01 33 00.

    - Product Data:  Submit product data for truck restraint equipment.

    - Shop Drawings:  Submit drawings indicating fabrication and erection of restraint equipment, including plans, elevations and large scale details.

      - Show anchorages, critical installation clearances, connections, and accessory items.

      - Provide location template drawings for items supported or anchored to permanent construction.

      - Provide rough-in drawings for electrical service in advance of concrete work.

    - Informational Submittals:  Submit following:

      - Manufacturer's instructions.

    - Closeout Submittals:  Submit following in accordance with Section 01 78 00.

      - Maintenance data.

      - Warranty:  Specified warranty.

  - QUALITY ASSURANCE

    - Single Source Responsibility: Each component of restraint device is required to be from same manufacturer.

    - Manufacturer Qualifications:  Company specializing in manufacturing Products specified in this Section with minimum ten years documented experience.

    - Installer Qualifications: Approved in writing by manufacturer with documented experience on at least five projects of similar nature in past three years.

    - Regulatory Requirements:  In compliance with ANSI MH30.3 AND applicable OSHA requirements.

    - Certifications:  Submit manufacturer's certification that products furnished for Project meet or exceed specified requirements.

    - Manufacturer to hold current ISO 9002 certification

  - WARRANTY

    - Special Warranty:  Prepare and submit in accordance with Section 01 78 00.

      - Standard manufactures warranty.

- - PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturers:

      - 4Front Engineered Solutions - Kelley, 1612 Hutton Drive, Suite 140, Carrollton, Texas, (972) 466-0707 voice, (972) 323-2661 facsimile

  - TRUCK RESTRAINT DEVICES

    - General:

      - Provide manufacturer's standard device designed to engage truck's rear wheel and hold truck at loading dock.  Driveway mounted restraint shall have a hot dip galvanized finish. System to be manually activated, to adjust to varying axle placement of a truck or trailer. Unit to be drive mounted with 4” (100 mm) ground clearance. Includes LED interior and exterior visual communication system and exterior audible alarm.

    - Operation: Truck is backed into position against the dock bumpers and the brakes are set. Outside light should be green and inside light should be red with the selector switch in the AUTOMATIC position. Operator moves the restraint carriage along the guide rail using activation handle to position the chock in front of the trailer wheel. Operator then extends restraining arm by pushing the handle toward the trailer until it locks into position on the guide rail. As restraining arm begins to extend toward the trailer, the outside light will change from green to red. Operator then moves the restraining arm backward to make contact with the trailer wheel. When the restraint has fully engaged the wheel, the inside lights will change from red to green. When loading/unloading is complete, the dock worker presses the RESTRAINT RELEASE BUTTON on the inside control panel. The inside light will change from green to red. The outside light will change from flashing red to flashing red and green. The operator then pushes the foot pedal, allowing the restraining arm to retract to its stored position. The inside light will remain red and the outside light will turn from red and green to flashing green, indicating it is now safe for the vehicle to leave.

    - Lights, alarm and switches on exterior of building not to exceed 24 volts.

    - Mounting:  Provide manufacturer's standard mounting method as follows

      - Driveway Mounted:  Mount unit to concrete allowing 4” clearance from ground to underside of rail.

    - Controls:  Control box to be operated by solid-state interlock capable controller with diagnostic capability.

    - Communication System:  Provide manufacturer's standard system, as follows:

      - Warning Signs: Set of surface-mounted signs.

      - Signal Lights: Set of illuminated LED exterior and interior signal lights to indicate device's status to both dock worker and restraint operator. Lights shall be activated automatically when device engages wheel.

      - Control box to be equipped with 3-light system to include Red, Green & Amber light system

      - Alarm:  Audible and visual system indicating that rear wheel is not engaged

      - Acceptable Product:  Model no.

        - 4Front Engineered Solutions - Kelley KMWC

  - FINISHES

    - Finish:

      - Preparation:  Clean surfaces free from slag and splatter, loose mill scale, oil, grease, or rust.

      - Factory applied manufacturer's standard hot dipped galvanized finish.

- - EXECUTION

  - EXAMINATION

    - Examine conditions and proceed with Work in accordance with Section 01 40 00.

  - INSTALLATION

    - Install in accordance with Section 01 60 00.

    - Connect electrical leads with control units.

    - Remote Control Station: Locate where indicated.

  - ADJUSTING AND CLEANING

    - Adjust installed unit for smooth, safe, efficient and balanced operation.

    - Remove temporary labels and coverings and protection of adjacent work areas.

    - Repair or replace damaged products.

    - Remove construction debris from site and dispose.

  - INSTRUCTION OF OWNER’S PERSONNEL

    - Instruct Owner’s personnel in operation and maintenance of installed units.  Provide bound copy of manufacturer’s operation and maintenance manual at time of instruction.

END OF SECTION