GUIDE

## SPECIFICATION

MANUFACTURER:

4Front Engineered Solutions - Kelley

US Corporate Office

# 1612 Hutton Drive, Suite 140

# Carrollton, Texas  75006

(972) 466-0707 voice

- (972) 323-2661 facsimile

- www.kelleycompany.com

SECTION 11 13 19

Master Control Panels

**********************************************************************************************************************************

This guide specification section has been prepared by 4Front Engineered Solutions – Kelley, and is intended to assist the specifier in preparing a specification for Control Panel operating multiple equipment operations from one control integrated panel.

Edit entire section to suit project requirements.  Modify or add items as necessary.  Delete items which are not applicable.  Words and sentences within brackets [_____] reflect a choice to be made regarding inclusion or exclusion of a particular item or statement.  This section may include performance, proprietary, and descriptive type specifications.  Edit to avoid conflicting requirements.

This guide specification is written around the Construction Specifications Institute (CSI), Section Format, and standard references to section names and numbers are based on the CSI MasterFormat 2004.

For specification assistance on specific product applications or information about other Kelley products, please contact our offices above or any of our local product representatives throughout the country.

4Front Engineered Solutions – Kelley reserves the right to modify these guide specifications at any time.  Updates to this guide specification will be posted to the manufacturer’s web site and/or in printed matter as they occur.  4Front Engineered Solutions makes no expressed or implied warranties regarding content, errors, or omissions in the information presented.

**********************************************************************************************************************************

- - GENERAL

  - SUMMARY

    - Related Documents: Provisions established within General and Supplementary Conditions of the Contract, Division 1 - General Requirements and Drawings are collectively applicable to this Section.

    - Related Sections:

      - [Division 16 - Electrical: Connections to dock equipment.]

  - SYSTEM DESCRIPTION

    - Single Source Responsibility:  Each component of dock leveler is required to be from same manufacturer.

    - Manufacturer Qualifications:  Company specializing in manufacturing Products specified in this Section with minimum ten years documented experience.

    - Installer Qualifications: Approved in writing by manufacturer with documented experience on at least five projects of similar nature in past five years.

    - Regulatory Requirements:  In compliance with ANSI MH14.1-1987 plus applicable OSHA requirements.

    - Certifications:  Submit manufacturer's certification that products furnished for Project meet or exceed specified requirements.

- - PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturers:

      - 4Front Engineered Solutions – Kelley, 1612 Hutton Drive, Suite 140, Carrollton, Texas, (972) 466-0707 voice, (972) 323-2661 facsimile

  - Products

- General:  Provide Single [Dual] panel to accommodate electronic function of all dock products for 1 [2] dock door(s). To include but not limited to levelers and truck restraints (Light Communication Package). A dedicated button shall be provided for each of the dock equipment listed.  Master control panel to be 100% UL and CSA approved (not just components).

- [Fused Disconnect]

- [15 amp Duplex Outlet]

- [Restraint Cycle counter]

- [Inflatable Shelter]

- [Door Heater/Blower]

- [Powered Door Controls to include Up, Down Stop buttons]

- [Dock Light Power]

- [Dock Light on/off switch]

- [Dock Light on/off automatic interconnect]

  - Type

- Control panel to be NEMA 12, automatic motor starter, thermal overload, 2-amp control breaker with reset capability.  All components to be individually circuit protected. S.A. and/or U.L. approved. [Mushroom style “Stop” button shall be standard].  [The “Stop” button shall cease all dock devices when depressed, and shall not require continuous pressure.]

- [208] [240] [480] volt, 3 phase motor

  - Functions

- Unit shall be capable of being programmed to control all associated dock equipment, and interlocked all equipment in sequence.

- - EXECUTION

  - EXAMINATION

    - Examine conditions and proceed with Work in accordance with Section 01 60 00.

  - ADJUSTING AND CLEANING

    - Adjust installed unit for smooth, safe, efficient and balanced operation.

    - Remove temporary labels and coverings and protection of adjacent work areas.

    - Repair or replace damaged products.

    - Remove construction debris from site and dispose.

  - INSTRUCTION OF OWNER’S PERSONNEL

    - Instruct Owner’s personnel in operation and maintenance of installed units.  Provide bound copy of manufacturer’s operation and maintenance manual at time of instruction.

END OF SECTION